# Table of Contents

Contents

Table of Contents	1

Important Links	2

SmartPac 3 wiki	2

SmartPac 5 wiki	2

1.16 - Gripper Adv/Ret Cycle Fault	3

Material Setup	4

Kickout “The scanned box barcode is not valid”	6

HMI not showing data after update	7

ASIN Tote Supply conveyor empty and/or Machine Starved warnings on HMI	8

# Important Links

## SmartPac 3 wiki

- https://w.amazon.com/index.php/FCShip/SmartPac

## SmartPac 5 wiki

- https://w.amazon.com/bin/view/FCShip/SmartPac/Version5_x/

# 1.16 - Gripper Adv/Ret Cycle Fault

- Indicates the gripper advance (move to right) cylinder advance reed switch is not working

- It is suggested that this sensor be repaired

- If you choose not to repair the sensor, enable g_par.24

# Material Setup

Only the config for the material you are running should be active (ON).  Typically, this is g_par_FilmWidth30in as shown in the screen shot below

# Kickout “The scanned box barcode is not valid”

# HMI not showing data after update

Update HMI firmware

https://drive.corp.amazon.com/folders/SmartPac/Code%20Repository/02_HMI%20Code/SmartPac%203.x/firmware

# ASIN Tote Supply conveyor empty and/or Machine Starved warnings on HMI

This is likely due to sensors being light activated vs dark activated.

- gPar.7 and gPar.8 allow inverting the sensor if needed. https://w.amazon.com/index.php/FCShip/SmartPac#g_Par_Definitions_.28Machine_Option_Settings.29

- Setup Information

https://w.amazon.com/index.php/FCShip/SmartPac#OEE_Starved_Condition_Definition