# Table of Contents

Contents

Table of Contents	1

Important Links	2

SmartPac 3 wiki	2

SmartPac 5 wiki	2

1.16 - Gripper Adv/Ret Cycle Fault	3

Material Setup	4

Ultrasonic sensor version (for 3.x machines)	6

Kickout “The scanned box barcode is not valid”	7

HMI not showing data after update	8

ASIN Tote Supply conveyor empty and/or Machine Starved warnings on HMI	9

# Important Links

## SmartPac 3 wiki

- https://w.amazon.com/index.php/FCShip/SmartPac

## SmartPac 5 wiki

- https://w.amazon.com/bin/view/FCShip/SmartPac/Version5_x/

# 1.16 - Gripper Adv/Ret Cycle Fault

- Indicates the gripper advance (move to right) cylinder advance reed switch is not working

- It is suggested that this sensor be repaired

- If you choose not to repair the sensor, enable g_par.24

# Material Setup

Only the config for the material you are running should be active (ON).

Toggle the g_par bit to indicate the width of SmartPac film running on the machine, i.e. whether the machine is running 24” wide bubble film, 30” wide bubble film, 36” wide bubble film (g_par.13, g_par.14, g_par.15 respectively) or if the machine is running SmartPoly film (g_par.16).

NOTE: Only one bit out of g_par.13, g_par.14, g_par.15, g_par.16 will be active at any time.

# Ultrasonic sensor version (for 3.x machines)

Toggle the g_par bit to indicate the IO Link version of ultrasonic sensor installed on the machine (g_par.18)

IO Link version 1.0 – Pico+25/WK/F, Pico+35/WK/F

IO Link version 1.1 – Pico+25/WK/F/A, Pico+35/WK/F/A

Front sensor model number (close to operator) : Pico+25/WK/F   Rear sensor model number (away from operator): Pico+35/WK/F     -   IO Link version 1.0 (g_par.18 ON)

Front sensor model number (close to operator): Pico+25/WK/F/A     Rear sensor model number (away from operator): Pico+35/WK/F/A – IO Link version 1.1 (g_par.18 OFF)

Note sensors have to be either both IO Link version 1.0 or both IO Link version 1.1.

# Kickout “The scanned box barcode is not valid”

# HMI not showing data after update

Update HMI firmware

https://drive.corp.amazon.com/folders/SmartPac/Code%20Repository/02_HMI%20Code/SmartPac%203.x/firmware

# ASIN Tote Supply conveyor empty and/or Machine Starved warnings on HMI

This is likely due to sensors being light activated vs dark activated.

- gPar.7 and gPar.8 allow inverting the sensor if needed. https://w.amazon.com/index.php/FCShip/SmartPac#g_Par_Definitions_.28Machine_Option_Settings.29

- Setup Information

https://w.amazon.com/index.php/FCShip/SmartPac#OEE_Starved_Condition_Definition