# Table of Contents

Contents

Table of Contents	1

Important Links	2

SmartPac 3 wiki	2

SmartPac 3 gPar setting	2

SmartPac 5 wiki	2

SmartPac 5 gPar settings	2

Material Setup	3

I/O Link version or Item Too Large - Please Remove Item From Bag And Cycle Machine Without Product Once	6

Ultrasonic sensor version (for 3.x machines)	6

1.16 - Gripper Adv/Ret Cycle Fault	7

Kickout “The scanned box barcode is not valid”	8

HMI not showing data after update	9

ASIN Tote Supply conveyor empty and/or Machine Starved warnings on HMI	10

# Important Links

## SmartPac 3 wiki

- https://w.amazon.com/index.php/FCShip/SmartPac

## SmartPac 3 gPar setting

- https://w.amazon.com/bin/view/FCShip/SmartPac#g_Par_Definitions_.28Machine_Option_Settings.29

## SmartPac 5 wiki

- https://w.amazon.com/bin/view/FCShip/SmartPac/Version5_x/

## SmartPac 5 gPar settings

- https://w.amazon.com/bin/view/FCShip/SmartPac/Version5_x/#Hg_ParDefinitions28MachineOptionSettings29

# Material Setup

Check the configuration on the HMI

- only one (1) box should be green

- options are read-only – changes must be made through the PLC as noted further down

SmartPac 3 HMI

SmartPac 5 HMI

Only the config for the material you are running should be active (ON).  Typically, this is g_par_FilmWidth30in as shown in the screen shot below

# I/O Link version or Item Too Large - Please Remove Item From Bag And Cycle Machine Without Product Once

- Visually verify ultrasonic sensor version – sensors are under cover above induct pocket

- Set g_par.18 to match the sensor version

  - IO Link 1.0

    - Pico+25/WK/F, Pico+35/WK/F

    - g_par.18 ON (1)

  - IO Link 1.1

    - Pico+25/WK/F/A, Pico+35/WK/F/A

    - g_par.18 ON (1)

# Ultrasonic sensor version (for 3.x machines)

Toggle the g_par bit to indicate the IO Link version of ultrasonic sensor installed on the machine (g_par.18)

IO Link version 1.0 – Pico+25/WK/F, Pico+35/WK/F

IO Link version 1.1 – Pico+25/WK/F/A, Pico+35/WK/F/A

Front sensor model number (close to operator) : Pico+25/WK/F   Rear sensor model number (away from operator): Pico+35/WK/F     -   IO Link version 1.0 (g_par.18 ON)

Front sensor model number (close to operator): Pico+25/WK/F/A     Rear sensor model number (away from operator): Pico+35/WK/F/A – IO Link version 1.1 (g_par.18 OFF)

Note sensors have to be either both IO Link version 1.0 or both IO Link version 1.1.

# 1.16 - Gripper Adv/Ret Cycle Fault

- Indicates the gripper advance (move to right) cylinder advance reed switch is not working

- It is suggested that this sensor be repaired

- If you choose not to repair the sensor, enable g_par.24

# Kickout “The scanned box barcode is not valid”

# HMI not showing data after update

Update HMI firmware

https://drive.corp.amazon.com/folders/SmartPac/Code%20Repository/02_HMI%20Code/SmartPac%203.x/firmware

# ASIN Tote Supply conveyor empty and/or Machine Starved warnings on HMI

This is likely due to sensors being light activated vs dark activated.

- gPar.7 and gPar.8 allow inverting the sensor if needed. https://w.amazon.com/index.php/FCShip/SmartPac#g_Par_Definitions_.28Machine_Option_Settings.29

- Setup Information

https://w.amazon.com/index.php/FCShip/SmartPac#OEE_Starved_Condition_Definition