Initial Camera Setup

Setting the aperture and focus on the camera:

- Park the robot over the tote, so that the camera lenses is roughly 20.5inches above the tote edge. Turn on EOAT light by turning on DO[87] using the robot TP (I/O button > F1 Type > 3 Digital > F3 until it shows DO’s >scroll to DO 87 > toggle ON using F4)

- If iRVision software has not been installed, do that at this time

  - Locate red usb stick inside robot controller, insert into usb drive of controller

  - Connect to the robot via web browser (11.200.1.20)

  - Click on “Vision Setup” tab

  - Install when prompted in internet explorer

- Using Internet Explorer, open Vision Setup and select SIDEA_CORNORLOATION program.

- Select LIVE view, and adjust the focus and aperture on the camera until you get an image that resembles the picture below. Note: make sure the tote is in the same orientation as the image below

- Stop the LIVE image, and press SNAP+FIND. If the camera is setup correctly, this tool should locate the corners of the tote, and outline them in GREEN. You will also see a score at the bottom of the screen.

- If the camera fails to find the corners, try adjusting the aperture or focus until you get a good image.