Camera Calibration

Setting up Calibration Grid User Frame

- While the robot is in STEP mode(press STEP button on TP, indicator should be yellow in status bar on top of TP screen), run the JR_Calibration program on the teach pendent. Stop at Point 1. The robot should be over the grid.

- With the robot and grid in position, you can run the Automatic Grid Frame Set function on the robot teach pendent.

  - Menu

  - IRVision

  - Vision Utilities

  - Automatic Grid Frame Set

  - Detail (F3)

  - Arrow down the Start Position, and press Record (F4)

  - Enter the values shown below for items 1-12 (Note: these values are just starting points, they may need to be adjusted)

- Once the starting position has been recorded, and all values set, you can execute the grid frame set function. CAUTION, Robot is going to move once you start!

  - Make sure Start Position is not highlighted.

  - While holding the enable switch and shift, press execute. (FCTN Abort All)

  - Once complete, the user frame will be taught for the Calibration Grid.

Camera Calibration

- While in STEP mode, Re-Run the JR_Calibration program on the teach pendent. Stop at Point 1.

- Using Internet Explorer, open Vision Setup and select TOTECAMERA program.

- On the Calibration Tab, SNAP an image, then select FIND for 1st PLANE.

- Using the Teach Pendent, STEP to point 2.

- On the Calibration Tab, SNAP an image, then select FIND for 2nd PLANE.

- Select SET.

- SAVE

- END EDIT

Camera Calibration Check

- While in STEP mode, Run the CalibrationCheck program on the teach pendent. Stop after the vision lights turn on.

- Using Internet Explorer, open Vision Setup and select CalibrationVerification program.

  - Click “Snap + Find”

  - Train this L-shaped dot pattern

    - Select “GPM Locator Tool”

    - Press “teach”

  - Expand the box around these 6 dots, then hit OK

  - Test it by hitting SNAP+FIND multiple times, make sure it finds it (we don’t care about the actual values)

  - Adjust the search window if necessary, should be similar to this pic

  - Retrain the single dot:

    - Select “Blob Locator Tool 1”

    - Select “Teach”, draw the box around the single dot, then hit OK

    - Adjust the search window so it is similar to the one shown below

    - Test this 30 times by hitting SNAP+FIND. Again we don’t care about the values, just that it found it. Do not use continuous S+F

  - Select 2-D Single View Vision Process, Snap and find multiple times to make sure everything works correctly

  - Click “Set”

  - These numbers update, they should be pretty close to zero (between -2 and 2). We only care about the X and Y values. We will use these values in “k”

  - Click “Save”

  - Take the Refence X and Y from “i” and manually enter them into the robot program CalibrationCheck, lines 11 and 12

  - Click End Edit in Internet Explorer

- Run the CalibrationCheck program multiple times to verify functionality. This program takes the values entered in Step3, sets limits of value +- 10, and verifies once a day that this trained position is within tolerance