Setting User Frames Using Automatic Grid Frame Set

Make sure the camera has been focused, and aperture has been set (see other document)

Pallet Frames Right Side:

- Run the “JR_CALIRIGHTSIDE” TP program. This will move the robot EOAT over Pallet Position 7.

- Load a pallet at position 7 and place the Calibration Plate on the pallet. Make sure that the X, Y orientation matches the robot world frame (images below).

- With the robot and grid in position, you can run the Automatic Grid Frame Set function on the robot teach pendent.

  - Menu

  - IRVision

  - Vision Utilities

  - Automatic Grid Frame Set

  - Detail (F3)

  - Arrow down the Start Position, and press Record (F4)

  - Enter the values shown below for items 1-12

- Once the starting position has been recorded, and all values set, you can execute the grid frame set function. CAUTION, Robot is going to move once you start!

  - Make sure Start Position is not highlighted.

  - While holding the enable switch and shift, press execute.

  - Once complete, the user frame will be taught for Pallet POS 7.

- Now that the frame is taught, you need to record the Master Height. Run the Rightside_Height program.

Pallet Frames Left Side:

- Run the “JR_CALILEFTSIDE” TP program. This will move the robot EOAT over Pallet Position 4.

- Follow steps 2 and 3 from above. Load Pallet into position 4, with grid.

- With the robot and grid in position, you can run the Automatic Grid Frame Set function on the robot teach pendent.

  - Menu

  - IRVision

  - Vision Utilities

  - Automatic Grid Frame Set

  - Detail (F3)

  - Arrow down the Start Position, and press Record (F4)

  - Enter the values shown below for items 1-12

- Repeat step 4 from above. Once complete, the user frame will be taught for Pallet POS 4.

- Now that the frame is taught, you need to record the Master Height. Run the Leftside_Height program.

- Lastly, Run the JR_UFCONFIG TP program to set the rest of the Pallet User Frames.