Camera Master Image

Setting up master image

- Allow robot to pick and place 5 totes from conveyor onto empty pallet with emulator

- On tote 6 (layer 2), stop the robot over the pallet with tote in gripper. Press step

- In step STEP mode, continue to step the robot program (place_pallet) thru line 75:

  - 75:L PR[11:Pallet Place] max_speed FINE Tool_Offset,PR[30:Vision Pic Off]    ;

- Skip down and run line 86-88 in step mode

  - 87:  R[75:Vision X Found]=0    ;

  - 88:  R[76:Vision Y Found]=0    ;

  - 89:  R[77:Vision R Found]=0    ;

- Skip down and run line 112 in step mode

  - 112:  IF R[4:Tote Position]<4,JMP LBL[4] ;

- Run to line 140 (186), but stop if it is going to crash into tote below

  - L PR[11:Pallet Place] max_speed CNT25 Tool_Offset,PR[25:Vision Place O] Offset,PR[18:Place -Z Offset] AP_LD15

- Center tote on layer 1 directly below tote in robot gripper so that it would place perfectly. Jog Z- slowly in world mode if the tote is not close enough (it should be just above the step) of the tote below it.

- After lined up perfectly, run program backwards (BWD instead of FWD button) to the previous point (Line 75)

- Using Internet Explorer, navigate to the robot at 11.200.1.20

- Enter “Vision Setup”.

- If iRVision software has not been installed, do that at this time

  - Locate red usb stick inside robot controller, insert into usb drive of controller

  - Connect to the robot via web browser (11.200.1.20)

  - Click on “Vision Setup” tab

  - Install when prompted in internet explorer

- Open the camera program, either SideA_CornerLocation or side B, depending what side of the tote is under the camera:

- Train the camera:

  - Click “Snap + Find”, green scribe lines should show on the image

  - Click “Set” to set the reference positions. This is the master offset the camera will use

  - Click “Save”

  - Click “End Edit”

- Do this same procedure for the SideA_EdgeLocation (or SideB depending on step 12) camera program [If EdgeLocation fails, try using a different tote] we are not doing this anymore

- Test the results of this by letting the robot continue to pick and place totes to fill up layer 2. There should now be 10 totes on the pallet

- For the first tote on layer 3, repeat this entire process for the opposite side of the tote

- Test the results of this by letting the robot continue to pick and place totes to fill up layer 2. There should now be 15 totes on the pallet