- From teach pendent press Select, then select the program JR-CALICRATION, press enter.

- Press the step button the teach pendent. The Step icon in the top left corner will turn amber.

- While holding the blue shift button, and both dead man switches on the back of the teach pendant, push and release FWD.

The program will step to the next line every time the FWD button is pushed. There will be a @ symbol beside the step you are on.

A prompt may appear saying that there is another program running. If so press the FCTN button and then press enter on the defaulted first option ABORT ALL

- Step though the program until the @sign is beside line 7 P (1) 25% Fine.

The robot should be positioned over the grid beside the infeed conveyor with camera lights on.

- After the robot has been moved to step 7, press menu and scroll down to 8 IRVISION. Select Vision setup from the submenu then enter.

- From the vision set up screen press the right arrow moving from #1 camera set up, to #2 calibration, then enter.

- Press F3 for Snap

- Press the 1st Plane Find button on the screen

There will be a prompt saying that there was a problem, disregard and move on.

- Press the EDIT button. This will take you back to the JR Calcration program

- While holding the blue shift button, and both dead man switches on the back of the teach pendant in, push FWD until the @ sign id beside step 10 P (2) 100% fine

- After the robot has been moved to step 10, press menu and scroll down to 8 IRVISION. Select Vision setup from the submenu then enter.

- Press F3 for snap, Then 2nd Plane Find

There will be a prompt saying that there was a problem, disregard and move on.

The calibration is not complete until it is saved and the edit is ended.

- Press Next and a save button will appear on the bottom of the screen. Press save.

- Press PREV and then F5 for END EDIT.

The camera is now calibrated. Step #2 may not be needed but if there are tote placement problems move on to step #2 Grid frame calibration.