NOTE: All personnel performing this task must sign off on the Standard Work Instruction.  By signing this document, the signee acknowledges they understand the work instruction and safety procedures and agree to follow all the steps, required and conditions listed on the form.

NOTE: Should any previously unidentified work hazard arise during the course of work, all work must stop and the Instruction amended (including initialing by all personnel), or a new instruction must be created and signed.

| Fulfillment Center: |  | Date of Work to be Performed: |  |  | Regulatory Compliance Window: |  |  |  | NEW/REVISED: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DSM5 |  | TODAY |  |  | Per PM Compliance Window |  |  |  | NEW REVISED |
| Work Order/TT Number: |  |  |  |  |  |  |  |  |  |
| WORK ACTIVITY (Description): |  |  |  |  |  |  |  |  |  |
| SLAM DAILY SYSTEM CHECK |  |  |  |  |  |  |  |  |  |
| Development Team |  |  | POSITION/TITLE: |  |  | Notified (Name): |  | POSITION/TITLE: |  |
| BRYABURG |  |  | CSL |  |  |  |  |  |  |
| Reflective Vest Hard Hat Personal Fall Arrest equip Safety Glasses Bump Cap |  |  |  | Goggles Face Shield Hearing Protection Safety Shoes—Type: Composite |  | Air purifying respirator Supplied respirator PPE Clothing—Type: Arc Flash Clothing (NFPA70E) | Gloves—Type: Work gloves Gloves—Specify: Other—Specify: Other—Specify: Other—Specify: |  |  |
| REMINDER: Use the Hierarchy of Control (elimination, substitution, isolation, reduction, engineering controls, administration controls) to determine appropriate actions to eliminate/mitigate hazards |  |  |  |  |  |  |  |  |  |
| JOB STEPS: List all job steps in sequence of performance |  |  |  | POTENTIAL HAZARDS: Specify what the potentials hazards are associated with each job step |  | CRITICAL ACTIONS TO MITIGATE HAZARDS: What specific actions need to be taken to eliminate or mitigate potential hazards |  |  |  |
| Preparation: - Review "Electrical Safe Work Practices Requirements" documentation and "Amazon Electrical Work Policy" - (https://inside.amazon.com/en/MyWorkplace/InsideNAFCSafety/Pages/NAFCSafetyPolicies.aspx) - Follow Standard Work Instruction – Distribution panel breaker trip - Always follow OSHA, LOTO, FTHA, NFPA, and Amazon Policies before placing hands on equipment. - Adhere to all safety guidelines when using lifts and ladders if required. - Communicate to Operations & RME MM of pending work. - If operations are impacted, open SEV1 or SEV2 Trouble Ticket |  |  |  |  |  |  |  |  |  |
| Properly prepare work area and tools. | - Slips, trips and falls - Damaged or improper tools |  | - Inform all affected employees that work is going to begin. - Be aware of surroundings. - Inspect all tools for wear and damage prior to the work. |  |  |  |
| Release latches to remove labels. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard - Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Remove label tape and inspect media sensors. Clean with air. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Clean brass peel roller. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Inspect the pinch roller. Clean with isopropyl alcohol. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Inspect the peel roller. Clean with isopropyl alcohol. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Inspect the platen roller. Clean with isopropyl alcohol. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Inspect the print head. Clean with isopropyl alcohol. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Verify toggles are not loose and they are properly tensioned and aligned. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Clean and inspect tamp pad for damage and debris. Remove any adhesive or obstruction from air holes. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Visually inspect tamp, assist, blow and vacuum solenoid pressures. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Visually inspect print quality. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Feed 5-10 labels. Visually inspect label alignment to tamp. Adjust guide collars as necessary. Listen for noise indicating worn components. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Inspect air assist bar for any damage. Verify bar is at proper angle and position. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Visually inspect entire printer, note any issues. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Verify zero by flagging exit photo eye on scale. Zero if necessary. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Visually inspect photo eyes, reflectors, belts for debris or damage. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Zero profilers using HMI. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Visually inspect conveyor for worn or missing O-rings between idler and MDR rollers. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Visually inspect belt alignment |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |
| Inspect Intralox steerable wheels. |  |  |  | - Hazardous Energy - Working at heights - Pinch point (Hand/ finger) - Ergonomic - Drop Scale Weight - Laceration Hazard Struck by |  | - Be aware of surroundings - Wear gloves |  |  |  |

NOTE: What can go wrong?  How can someone get hurt?  Potential Hazards:  Impact, Penetration, Compression or Rollover, Chemical, Heat, Inhalation, Light/radiation, Electrical Control Hazard, Noise, Potential Fall, Asphyxiation